# Appendix C. QUICK REFERENCE HANDBOOK (QRH)

Airfield Administration & Contact Details

Name of aerodrome operator:  Caboolture Aero Club Incorporated

Postal address:  Lot 451 McNaught Road, Caboolture, Queensland 4500

Phone number:   	0488 922 245

E-mail address:   	secretary@cacq.com.au

Website:   	www.cabooltureaeroclub.com.au

Airfield usage:   	Private & Public Use. Visitors welcome and public use

available with Prior Permission Required. (PPR)

YCAB IS ONLY TO BE USED DURING DAYLIGHT HOURS– CTAF FREQ 125.85 MHZ

Airfield Location

Aerodrome name:   	 Caboolture Airfield

State/Territory:   	 QLD

ARP latitude (WGS84):   	 	270437S

ARP longitude (WGS84):   	 	1525913E

Y location code:   	YCAB

Elevation:  	 	40 ft.

SUMMARY OF SIGNIFICANT FLIGHT PROCEDURES

The following Rules are the most significant and important rules which if properly followed will provide the safest and consistent flying environment at Caboolture Airfield.

CASA Part 91 applies to all categories.

- Right Hand Circuits are required when operating Runway 29.

- NO Simultaneous Runway Operations are permitted under any circumstances.

- Under light or variable wind conditions, Runway 11 is the designated preferred runway due to Noise Abatement.

- The change of Active Runway Procedure;

Pilot requiring a change of runway due Crosswind or wind direction change, communicates by radio to ALL aircraft in the circuit or taxiing, and must receive confirmation or acknowledgement from ALL aircraft to accept the change. ALL aircraft in the circuit are allowed to complete the current circuit and land or depart the circuit. No aircraft are to take off prior to ALL aircraft in the circuit completing the landing and hearing a radio call of “Clear/Vacated” or “Departed the Circuit.”

- A “Rolling” call must be made at the commencement of take-off.

- Fly Neighbourly requires complete knowledge of Noise Sensitive Areas (NSA) and circuit procedures for Runways 29 and 24. Visiting pilots will be warned or banned if violation is repeated. (Refer ERSA)

- Intersection departures are NOT permitted for all aircraft types except helicopters. Helicopters must taxi to intersection and depart using active runway direction in use at all times.

- All aircraft including helicopters and rotorcraft to conform to standard circuit rules – AC 91- 10 applies.

- Straight-In Approaches are NOT Permitted on any Runway.

- Helicopters may operate to the side of all Runways, (outside gable markers) but where possible avoid centrelines due to damage to grass.

- An aircraft within the gable markers is deemed as occupying the runway strip.

- Helicopters are not to use sealed taxiway north of Hangars on Taxiway B; use the sides of Runway 06/24 instead.

- Helicopters not to taxi between hangars.

- See and Avoid Procedures and Situational Awareness require effective Position Reporting by all pilots on the ground and in the circuit. (91 MOS Table 21.04 through 21.07)

- Rules for Taking off and Landing procedures (CASR 91.370) must be adhered to.

- A maximum of five (5) aircraft conducting circuit training in the circuit at any one time.

- All surface vehicles (lights illuminated) are to make a Radio call prior to entering or crossing an Active Runway.

- No Land and Hold Short Operations (LAHSO) to be conducted at any time.

- Runway 24 not to be used, except for emergency or inclement weather – Left turn at 500 ft. AGL.

IF AN EMERGENCY OCCURS AT CABOOLTURE AIRFIELD, WHAT DO YOU DO?

CLUB PRESIDENT ONLY – IS AUTHORISED TO COMMUNICATE WITH MEDIA

ASSESS THE SITUATION – CHECK THE FOLLOWING DETAILS!

AIRCRAFT OR VEHICLE – NATURAL DISASTER – FIRE - UNLAWFUL ACTIVITY

CLEARLY IDENTIFY LOCATION

PERSONS INVOLVED - NUMBER - POSSIBLE INJURIES

HAZARDS – FIRE – CHEMICALS – BUILDING OR HANGAR INCIDENT

SAFE ACCESS TO INCIDENT – IDENTIFY EMERGENCY ASSEMBLY AREAS

SECURITY OF SITE

COMMUNICATE ASAP - CALL!

BUILDING - YELL OUT A WARNING TO EVACUATE - DO NOT LOCK DOORS

CALL 000 FOR ASSISTANCE – INCIDENT DETAILS - GIVE SITE ACCESS DETAILS

CALL CLUB EMERGENCY PHONE - 0488 922 245

FOLLOW INSTRUCTIONS FROM CLUB PRESIDENT, ARO OR TEAM LEADER

PROVIDE ASSISTANCE IF SAFE TO DO SO – CARE FOR INDIVIDUALS!

INJURIES - RENDER FIRST AID IF SAFE TO DO SO

USE FIRE EXTINGUISHER / FIRE HOSE

HAZARDS – CLEAR SAFETY DISTANCE/ENVIRONMENT – WAIT FOR EMERGENCY SERVICES

TEAM LEADER - FACILITATE & COORDINATE ACCESS FOR EMERGENCY SERVICES

CONFER WITH EMERGENCY SERVICES

DIRECT & SAFELY GUIDE SERVICES TO INCIDENT SITE – CHECK GROUND CONDITIONS

CLOSE THE SITE AIR OPERATIONS – REFUELLING OR MAINTENANCE- AS NEEDED

INSTALL X’S ON RUNWAYS & WINDSOCK CIRCLE - AS REQUIRED

COMMUNICATE – MONITOR ALL FLIGHT OPERATIONS – CTAF OR PHONE

APPOINT TASK PERSONS – SPECIFY DUTIES

MAINTAIN SITE SECURITY - CONTROL THE ACCESS GATES

ARRANGE FOR REPLACEMENT TASK PERSONS – IF LENGTHY INCIDENT RECOVERY

WHEN APPLICABLE, COMPLETE SITE RECOVERY OPERATIONS

CONFIRM SITE SERVICEABILITY

REOPEN THE AIRFIELD – FULL OR PARTIAL WHEN DEEMED SUITABLE

ONCE FULL RECOVERY OF AIRCRAFT OR VEHICLE OR HAZARD - COMPLETED

SITE INSPECTIONS HAVE BEEN COMPLETED – INCIDENT DETAILS RECORDED

SAFETY OF ALL CONDITIONS - DEEMED COMPLETED

POST INCIDENT REVIEW

TEAM LEADER TO CONVENE AT APPROPRIATE TIME – OFFSITE LOCATION

CHECK WITNESS STATEMENTS

DEBRIEF THOSE INVOLVED - INCLUDE EMERGENCY SERVICES

AERO CLUB MANAGEMENT COMMITTEE

REVIEW THE INCIDENT – EXPERIENCE GAINED – CONTACT AUTHORITIES IF

REQUIRED - REVISE & UPDATE EMERGENCY RESPONSE PLAN AS NEEDED

Runway bearing, length, width, and surface type

The bearings, length, width, and surface type(s) of the runway(s) is recorded in the table below:

| Runway | Runway bearing (Magnetic) | Runway length (m) | Runway width (m) | Runway surface type, or types (non-homogenous runways) |
| --- | --- | --- | --- | --- |
| RWY 11/29 | 120/300 | 1210m 970m TODA | 8m | Grass. First 400m (8 m wide) of RWY 11 sealed Bitumen. |
| RWY 06/24 | 058/238 | 820m 768m TODA | 8m | Grass |

Runway Pavement Strength rating

The runway(s) at Caboolture Airfield are natural surface runways without formed pavement except the first 400 m. of RWY 11 of sealed bitumen.   A ‘Traffic Light’ system has been adopted and can be found on the club website. https://www.cabooltureaeroclub.com.au/runway-status/

The Airfield is limited to 5,700KG and below. Emergency Service Aircraft are Exempt.

Fuel Suppliers

Fuel suppliers and their contact details are recorded in the table below:

| Fuel supplier | Fuel type | Contact details | After-hours contact details |
| --- | --- | --- | --- |
| VIVA: Unmanned 24HR Card Swipe Bowser | AVGAS | 0408 711 968 | N/A |
| Caboolture Aeroclub | MOGAS | 0488 922 245 | N/A |

Location of Runway threshold/s

The following runway thresholds are permanently displaced from the extremity of the runway:

| Runway end | Distance of threshold displacement | Reasons for permanent threshold displacement |
| --- | --- | --- |
| RWY 06 | 250m - Permanent | Displaced due to Noise Abatement Procedures Local Government Notice of Consent section 3(a)(i) |
| RWY 29 | 240m - temporary | Displaced due to Trees along boundary infringing on OLS on Eastern end |

NOTE – Pilots must be aware that the trees located between Runway 11/29 and Runway 06/24 may obstruct their ability to observe traffic during take-off and landing at the airfield. Pilot should use adequate and sufficient communication in accordance with CAS AC 91-10 to ensure safety of operations.                    BE SEEN, BE HEARD, BE SAFE.